Computer Networks Lab Report

Submitted By

SURIYA TS

RA2411030050051

Under the guidance of

Dr. Mallika

(Assistant Professor, School of Computing)

in partial fulfilment for the award of the degree

of

BACHELOR OF TECHNOLOGY

in

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING WITH CYBER SECURITY

Of

FACULTY OF ENGINEERING AND TECHNOLOGY

SRM INSTITUTE OF SCIENCE AND TECHNOLOGY,

TIRUCHIRAPPALLI

Academic Year: 2026–2027

Date: 23 October 2026

Computer Networks Lab – Experiment 1

Experiment Title

Introduction to Packet Tracer, Peer-to-Peer Communication, Study of Cables and Their Color Codes.

Aim

To get familiar with the Cisco Packet Tracer simulator, to set up a simple network of two PCs connected through a switch and verify communication between them using static IP addresses, and to study Ethernet cables and their T-568A/T-568B colour codes.

Software Requirements

- Cisco Packet Tracer (network simulation software)

- Windows PC to run the simulator (project saved as lab 1.pkt)

Brief Theory

Packet Tracer: a simulation tool used to design, configure and test networks using virtual devices such as PCs, switches and routers, without physical hardware.

Peer-to-peer communication: two hosts exchange data directly with each other as equals. Here, PC0 and PC1 are linked through a switch and, being in the same network (192.168.1.0/24), can communicate without a router. The ping command (ICMP echo request/reply) is used to test reachability.

Cables and colour codes: Ethernet UTP cables contain four twisted pairs (eight wires) terminated in an RJ-45 connector using either the T-568A or T-568B standard. A straight-through cable uses the same standard at both ends and connects unlike devices (e.g., PC to switch). A crossover cable uses T-568A at one end and T-568B at the other and connects like devices (e.g., PC to PC). The wire order is given below.

| Pin | T-568A | T-568B |
| --- | --- | --- |
| 1 | White/Green | White/Orange |
| 2 | Green | Orange |
| 3 | White/Orange | White/Green |
| 4 | Blue | Blue |
| 5 | White/Blue | White/Blue |
| 6 | Orange | Green |
| 7 | White/Brown | White/Brown |
| 8 | Brown | Brown |

Step-by-Step Procedure

- Open Cisco Packet Tracer and use the Logical workspace to build the network.

- From the device palette, place two PCs (PC-PT), named PC0 and PC1, and one 2960-24TT switch (Switch0) on the workspace (Fig. 1).

- From the Connections palette, select the Copper Straight-Through cable and connect PC0 to Switch0 and PC1 to Switch0. The link indicators on both cables turn green, showing that the links are up (Fig. 2).

- Click PC0, open Desktop > IP Configuration, select Static and enter IPv4 address 192.168.1.1 with subnet mask 255.255.255.0 on interface FastEthernet0 (Fig. 3).

- Repeat the configuration on PC1 with IPv4 address 192.168.1.2 and subnet mask 255.255.255.0 (Fig. 4).

- On PC0, open Desktop > Command Prompt and run ping 192.168.1.2 to test connectivity with PC1 (Fig. 5).

- Study the T-568A and T-568B colour codes of Ethernet cables (Fig. 6).

Fig. 1: PC0, PC1 and 2960-24TT Switch0 placed in the Logical workspace (no links yet)

Fig. 2: PC0 and PC1 connected to Switch0 using copper straight-through cables; link indicators are green

IP Address Configuration

| Device | Interface | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | FastEthernet0 | 192.168.1.1 | 255.255.255.0 | 0.0.0.0 |
| PC1 | FastEthernet0 | 192.168.1.2 | 255.255.255.0 | 0.0.0.0 |

Both PCs use static addressing (DHCP not selected). The default gateway and DNS server were left at 0.0.0.0, since both PCs are in the same network and no router is used.

Fig. 3: IP Configuration of PC0 (192.168.1.1 / 255.255.255.0)

Fig. 4: IP Configuration of PC1 (192.168.1.2 / 255.255.255.0)

Testing and Verification

Connectivity was tested from PC0 using the command ping 192.168.1.2 in the Command Prompt. The output in Fig. 5 shows the following:

- Four replies were received from 192.168.1.2, each with bytes=32, time<1ms and TTL=128.

- Packets: Sent = 4, Received = 4, Lost = 0 (0% loss).

- Round-trip time: Minimum = 0ms, Maximum = 0ms, Average = 0ms.

Fig. 5: Successful ping from PC0 to PC1 (192.168.1.2) with 0% packet loss

Study of Cables and Their Color Codes

Fig. 6 shows the pin-wise colour arrangement of the T-568A and T-568B standards. This is a theoretical study using the reference diagram; physical inspection or crimping of a cable is not part of the evidence in this report. In Packet Tracer, the Copper Straight-Through cable was used for both PC-to-switch links.

Fig. 6: T-568A and T-568B Ethernet cable colour codes (reference diagram)

Result

A network of two PCs (PC0 – 192.168.1.1 and PC1 – 192.168.1.2) connected through a 2960-24TT switch with copper straight-through cables was created in Packet Tracer. Ping from PC0 to PC1 returned 4 of 4 replies with 0% loss, confirming successful communication. The T-568A and T-568B colour codes were studied.

Conclusion

The experiment introduced the Packet Tracer interface and the steps of building a simple network, assigning static IP addresses and verifying connectivity with ping. Hosts in the same subnet communicate directly without a router, and a straight-through cable is used to connect a PC to a switch. The study of T-568A and T-568B colour codes explains how straight-through and crossover cables are wired.